# Receptionist Context Window Log Record

Generated: 2026-06-25 15:14:01 +08

Workspace: /home/yanjiun68/workspace/WS1/PutrabotUPM-project/src

Role context: ROS2 autonomous system developer

## Context Window Summary

This log records the explored decisions, implemented package work, and verification results from the current development window. The work focused on ROS2 navigation map and waypoint handling, SMACC2 state-machine inspection, and a new InsightFace feature-summary state machine that speaks detected appearance features through the existing TTS service.

The final implemented feature in this window is a new lifecycle-managed SMACC2 package named insightface_feature_summary_smacc2. It launches the InsightFace appearance pipeline, waits for a visible face, collects appearance messages, stores detected features in SMACC2 global data, generates flexible speech text, calls the TTS service once, and cleans up the launched InsightFace process tree.

[Context Window Work]

|

+--> Navigation map and waypoint usage

|

+--> Rviz/Nav2 goal pose and orientation discussion

|

+--> Named waypoint action flow

|

+--> SMACC2/RTA viewer inspection

|

+--> InsightFace feature-summary SMACC2 implementation

|

+--> TTS service-call design and verification

## Navigation And Mapping Work

The navigation discussion covered how static_map_nav.launch.py and mapping-related launch files are used in the robot navigation workflow. The map argument in the static navigation launch command selects the saved map YAML file loaded by Nav2 map server. That YAML points to the map image and defines metadata such as resolution, origin, occupied threshold, and free threshold.

For map export, the expected ROS2 workflow is to run SLAM or mapping, then save the map using the Nav2 map saver CLI:

ros2 run nav2_map_server map_saver_cli -f /path/to/map_name

This produces:

map_name.yaml

map_name.pgm

The YAML is then passed back into the static navigation launch file with a map:=... argument.

[SLAM / Mapping Running]

|

v

[map_saver_cli -f map_name]

|

v

[map_name.yaml + map_name.pgm]

|

v

[static_map_nav.launch.py map:=map_name.yaml]

|

v

[Nav2 map server publishes static map]

## Waypoint YAML And Named-Pose Action Flow

The waypoint workflow moved toward storing target poses in a YAML file so named locations can be reused without manually clicking in RViz every time. The current intended source is the active waypoint YAML under NAVIGATION/my_robot_nav/maps/, not stale waypoints from an older map.

A target pose in the map frame needs:

position:

x: <meters_in_map_frame>

y: <meters_in_map_frame>

z: 0.0

orientation:

x: 0.0

y: 0.0

z: <quaternion_z>

w: <quaternion_w>

For 2D navigation, the pose is usually interpreted as:

x, y, yaw

The orientation in RViz is represented internally as a quaternion. If only 2D coordinates are visible in a terminal output, the orientation can be checked by echoing the full goal pose topic/action payload or by converting quaternion z,w into yaw:

yaw = atan2(2*w*z, 1 - 2*z*z)

The named-pose action flow discussed in this window was:

ros2 action send_goal /nav/navigate_named_pose \

receptionist_interfaces/action/NavigateNamedPose \

"{target_name: 6}" \

--feedback

If the command remains at:

Waiting for an action server to become available...

then the named-pose action server is not running, has a different action name, or the workspace was not sourced correctly.

[waypoints.yaml]

|

v

[NavigateNamedPose action server loads names]

|

v

[ros2 action send_goal /nav/navigate_named_pose]

|

v

[Server resolves target_name to PoseStamped]

|

v

[Nav2 NavigateToPose action]

|

v

[Robot moves to waypoint]

## SMACC2 And RTA Viewer Investigation

The state-machine inspection focused on the receptionist SMACC2 package and the ability to view the state-machine flow. The referenced launch file was:

rc_task/sm_receptionist/launch/smacc2_receptionist.launch.py

The intended RTA viewer workflow is to launch the SMACC2 state machine with its introspection support enabled, then open the runtime analyzer/viewer to inspect states, transitions, and active orthogonals.

The existing insightface_camera_switch_smacc2 package was used as the reference style for lifecycle-supervised SMACC2 execution. It contains:

rc_task/insightface_camera_switch_smacc2/src/camera_switch_smacc2_node.cpp

rc_task/insightface_camera_switch_smacc2/src/camera_switch_lifecycle_node.cpp

rc_task/insightface_camera_switch_smacc2/include/insightface_camera_switch_smacc2/process_utils.hpp

That package pattern uses:

[Lifecycle node]

|

v

[Detached SMACC2 process]

|

v

[Detached InsightFace launch process]

|

v

[Process group registry in /tmp]

|

v

[Clean shutdown by process group]

## InsightFace Feature-Summary SMACC2 Package

A new package was implemented:

rc_task/insightface_feature_summary_smacc2

Created files:

rc_task/insightface_feature_summary_smacc2/CMakeLists.txt

rc_task/insightface_feature_summary_smacc2/package.xml

rc_task/insightface_feature_summary_smacc2/include/insightface_feature_summary_smacc2/process_utils.hpp

rc_task/insightface_feature_summary_smacc2/launch/feature_summary.launch.py

rc_task/insightface_feature_summary_smacc2/src/feature_summary_lifecycle_node.cpp

rc_task/insightface_feature_summary_smacc2/src/feature_summary_smacc2_node.cpp

The launch file exposes the following important defaults:

personal_test_mode:=true

start_tts:=true

tts_service_name:=/voice/tts/say

face_topic:=/perception/largest_face

appearance_topic:=/appearance/current_estimate

The SMACC2 flow is:

[StLaunchInsightFace]

|

v

[StWaitForFace]

|

v

[StCollectAppearance]

|

v

[StBuildSummary]

|

v

[StSpeakSummary]

|

v

[StStopInsightFaceAndFinish]

Failure from launch, face wait, appearance collection, summary generation, or speech output transitions to:

[StAborted]

|

v

[Stop registered InsightFace process group]

|

v

[Shutdown]

The state machine stores detected features in SMACC2 global data:

shirt_color

shirt_color_confidence

glasses_state

glasses_confidence

hat_state

hat_confidence

face_mask_state

face_mask_confidence

feature_summary_text

The appearance message used by the package is:

receptionist_interfaces/msg/GuestAppearance

The collected fields are:

shirt_color

glasses

hat

face_mask

confidence values for each feature

## TTS Service-Call Logic

The speech output uses:

voice_interfaces/srv/SayText

Service shape:

string text

---

bool success

string message

Default service name:

/voice/tts/say

The text generation is flexible because it is generated after appearance aggregation. The state machine chooses sentences based on detected, not detected, or unknown states.

Example output shape:

I can see a person wearing a blue shirt. I detect glasses. I do not detect a hat. I do not detect a face mask.

If the feature cannot be confidently determined, the generated text avoids overclaiming:

I can see a person, but I cannot confidently determine the shirt color.

I cannot confidently determine whether the person is wearing glasses.

The TTS service is called exactly once in the speech state:

[Aggregated appearance global data]

|

v

[Build feature_summary_text]

|

v

[CbSpeakFeatureSummary]

|

v

[One SayText request to /voice/tts/say]

## Build And Verification Results

The package was discovered by colcon:

colcon list --packages-select insightface_feature_summary_smacc2

The package and required dependencies were built from the workspace root:

colcon build --packages-up-to insightface_feature_summary_smacc2

Result:

6 packages finished

insightface_feature_summary_smacc2 built successfully

The installed executables were verified:

source /opt/ros/humble/setup.bash

source install/setup.bash

ros2 pkg executables insightface_feature_summary_smacc2

Result:

insightface_feature_summary_smacc2 feature_summary_lifecycle_node

insightface_feature_summary_smacc2 feature_summary_smacc2_node

The launch arguments were checked with:

ROS_LOG_DIR=/tmp/roslogs \

ros2 launch insightface_feature_summary_smacc2 feature_summary.launch.py --show-args

The check confirmed the expected defaults, including:

personal_test_mode default: true

start_tts default: true

tts_service_name default: /voice/tts/say

## Known Limitations And Next Steps

Full hardware runtime testing was not completed in this context window. Specifically, the camera stack, live InsightFace detection, physical robot motion, and audible Piper playback were not run end-to-end during verification.

Recommended next runtime tests:

source /home/yanjiun68/workspace/WS1/PutrabotUPM-project/install/setup.bash

ros2 launch insightface_feature_summary_smacc2 feature_summary.launch.py

For RealSense mode:

ros2 launch insightface_feature_summary_smacc2 feature_summary.launch.py personal_test_mode:=false

Runtime checks to perform:

ros2 topic echo /perception/largest_face --once

ros2 topic echo /appearance/current_estimate --once

ros2 service list | grep /voice/tts/say

If the state machine times out waiting for face or appearance data, check:

camera device availability

InsightFace model dependencies

topic names

workspace sourcing

ROS_DOMAIN_ID

TTS node availability

## Microsoft Word Compatibility Notes

This log is maintained as Markdown for version control and future editing. A .docx export is generated for Microsoft Word compatibility and user-manual preparation.

Recommended document policy:

Markdown file = editable source of truth

DOCX file = exported manual/reference copy

The .docx file should open in Microsoft Word and LibreOffice Writer. Future updates should be made in the Markdown source first, then exported again.